Top Level Schematic:

- Based on NCP1681AA EVB: 500 W @ 90 – 180 VAC, 1 kW @ 180 – 265 VAC

Component Models:

- Component models were developed for the current sense transformer (CT) and the CT blanking circuit to simplify the schematic

- Note that R21 (100 Ω) is not in real circuit but was necessary to improve convergence and speed of simulation

- The Current sense transformer (CT) modeled as “ideal transformer” (current controlled current source) with secondary side magnetizing inductance.  Magnetizing inductance and turns ratio are parameterized so customer can adjust values at top level to match their CT specifications

- Model is not encrypted so customer is free to modify this component

- The dual P-Channel FETs used in the blanking circuit are modeled with FDS9958 translated SPICE files that are native to Simplis.  This was done to reduce space on top level

- Similarly, the 2 series BAS16 diodes are modeled as single diode (D2) to reduce space.

- Neither model is encrypted so customer is free to modify these components

Input Current Measurement:

- Top level model did not use emulated EMI filter, instead input current measurement can be done directly with ILine probe and customer can model their EMI filter

- V11 is AC input current source.

- V2 can be used to test line voltage transient.  When not testing line voltage transient then set “Delay” input to be longer than simulation time and make sure that “Off until delay” box is checked

- If customer wishes to add emulated EMI filter, recommended circuit is shown below

- Recommended circuit is 2x 100 µs RC filters, R2/C2 and R11/C11.

Skip Mode Testing:

- V3 and S5 can be used to test Skip mode operation

- V3 is prepopulated with the 10 ms, 1% duty cycle, pulse train.

- If not testing Skip then add delay and click “Off until delay”

Output Voltage & Transient Response:

- Output voltage may exhibit high frequency ripple that is artifact of emulation and not necessarily real behavior.

- A filtered output voltage response is provided

- Transient or overload testing can be configured with V1, S4 and parallel combination of R2/R23.

Controller Model:

- Controller model is encrypted so cannot be accessed by user

- C2 = CP and C11 = CZ.   User can set initial conditions of the compensation capacitors to allow simulation to achieve steady state faster

- V21 is PWL source for VCC bias.